Hi everyone, finally after 2 weeks of intense studying we managed to install Cnav+ on my stilo MW in substitution of the old mp3 player. Hope to help anyone who wants to attempt this using my experience and parts used with the installation. I will help to the best of my ability, and in any case I can promise that it’s not absolutely difficult, but you will need a fair amount of ability in the dismantling/reassembly of the relevant parts of the car.

1) Obtain the Cnav+ unit and the code.

2) Buy the original FIAT bracket for mounting the nav+ unit. (I will post the part number later on) 11 euros  (possible part number-46790149)

3) Buy the roof light unit console, included in this is a preamplified microphone (part no- 735303207 without other push-buttons for skyroof, alarm. etc.) 55 euros

4) Buy Multifunction Antenna. In my case I bought THIS, the Calearo mod Triplex cod. 7680210 and two adapters: cod. 7581066 - cable adapter in order to connect the radio cable to the Fiat radio cable. (ATTENTION: in case the connection between the Calearo-Fiat connectors is difficult, remove the reduction ring from the inside of the Calearo connector with a thin screwdriver).

cod. 7581077 - cable adapter for GPS between calearo cable and grey connector plug (gt5 F) (GPS) of the connect Nav+.

(Note the antenna FM/AM cable is not included because the original is used) Cost of antenna and 2 adapters 90 euros

5) FASTON (Spade) connector for ISO connector. (Needed for powering PIN4 of the original ISO connector when key is on. This enables the Connect Nav+ to link to the CAN)

6) You will need a lot of patience and good luck!

Let’s begin:

Remove the surrounds and relevant parts around the area to be worked on.

Be careful as there are some fiddly plastic dowels when removing the plastic storage tray.

I also removed the panel to the side that incorporates the passenger airbag disabling switch.

Finally after removing these parts, we can now remove the old radio bracket. There are torx screws around it, and a hex screw on the inside of the bracket. It will not come off yet as it is trapped by the plastic around the air con control and under the radio. Unscrew all the torx screws and gently extract the old radio bracket.

Now you can insert and secure your new bracket.

Installing the microphone: Dismantle the roof lights console, and somewhere inside you should find some unused cables with connectors wrapped with tape. One of these will be for the microphone (two pole long black connector with a BLUE wire and a WHITE&ORANGE wire) Connect this plug to the microphone and reassemble with new console. (Note: The microphone cables go to a white connector (D020) in the fuse box area down near the steering wheel.

Now to mount the antenna:

Remove the rubber seal from around the rear hatch/tailgate opening.

Remove the roof lining retainers under the plastic covers.

Remove the grab handles on the left and right.

Pull the top sections of the door seals down.

This should enable you to access the old antenna for removal, and replacing with the new one.

Connect the existing AM/FM cable to the new antenna part with the adaptor. We also connect the GPS and GSM cables to the new antenna part and begin to pass these last ones along the left edge of the roof that we have previously disassembled.

Feed the cables under the covering of the left hand mounting which is not necessary to detach, and then through the glovebox space, eventually arriving behind the connect unit.

We now come to the connection of the microphone cable.

Use a two wire shielded cable to connect to the D020 connector (mentioned above) you can use faston(spade) connectors, or solder them (as I have done).

Feed the cable behind the steering column and route it up to the Connect unit.

Now we need to obtain a 12V feed that works when key is on. (I have taken it from the cigarette lighter feed. Passing the wire to it through the tunnel/console) take this wire up behind the Connect unit. An inline fuse should be used on this wire.

At this point I reassembled the roof lining, door seals, and other parts removed for access.

Now all that remains is to carry out the connections behind the Connect unit. Connect jack of FM antenna through adapter with the grey GPS connector.

I couldn’t find an adaptor anywhere for the Connect unit’s GPS socket (green one), so I soldered the cable directly into the socket. A bit rough maybe, (but it works like a dream).

The 12V switched power lead goes to PIN4 of the ISO connector that we have. (The black thing).

Finally the leads of the made up cable for the microphone coming from the D020 connector are connected to the P20F connector of the Connect unit (white, lower left PIN9 & PIN18) either by soldering or spade connectors.

Now we can put the Connect unit in and reassemble everything we took apart before. Everything should function as it should…except for the annoying flashing of the total mileage. But.. LISTEN LISTEN! We are working on this to find a solution, based on inside information we are getting from Magneti Marelli and FIAT. Be patient, sorry about this but it was better if you know.